Covered Deck

The deck consists of 13.1 sqft. of commercial fiberglass deck grating.

At 40 lb/sqft, the total load is 524 lbs.

The deck is supported by plywood/epoxy gusset beams that are in plane of and bonded to the adjacent sides.

Each gusset must support half the load along 2’ of length, 131 lbs per foot or 11 lbs per inch.